CE-580

COMPUTATIONAL TECHNIQUES

FOR

FLUID DYNAMICS

HOMEWORK #7

Vorticity-Stream Function Solution to Driven Cavity Flow

Taha Yaşar Demir

1881978

# Governing Equations

For a 2-D incompressible, viscous flow in x-y

Vorticity vector written as ζ=∂v∂x-∂u∂y and vorticity transport equation as

∂ζ∂t+u∂ζ∂x+v∂ζ∂y=ν∂2ζ∂x2+∂2ζ∂y2

Or in conservation form

∂ζ∂t+∂(uζ)∂x+∂vζ∂y=ν∂2ζ∂x2+∂2ζ∂y2

Velocity components can be written in terms of stream function

u=∂ψ∂y ,  v=-∂ψ∂x

Poisson equation for stream function can be obtained from the vorticity component in x-y plane

∂2ψ∂x2+∂2ψ∂y2=-ζ

Now we have one parabolic and one elliptic equation to solve

# Discretization

Vorticity transport equation is discretized with First-Order Upwind method. This method is stable and dissipative but introduces artificial viscosity. Thus, a fine mesh should be used.

ζi,jn+1-ζi,jnΔt+(1-ϵx)2uζi+1,jn-uζi,jn Δx+(1+ϵx)2uζi,jn-uζi-1,jn Δx+(1-ϵy)2vζi,j+1n-vζi,jn,Δy+(1+ϵy)2vζi,jn-vζi,j-1nΔy=νζi+1,jn-2ζi,jn+ζi-1,jnΔx2+ζi,j+1n-2ζi,jn+ζi,j-1nΔy2

If ui,j>0, backward difference must be used Thus, ϵx=1

If ui,j<0, forward difference must be used Thus, ϵx=-1

If vi,j>0, backward difference must be used Thus, ϵy=1

If vi,j>0, backward difference must be used Thus, ϵy=-1

If ϵx=ϵy=0,second-order central differences are recovered

Poisson equation for stream function is discretized using second order central differences and yields FDE such as

ψi41,j-2ψi,j+ψi-1,jΔx2+ψi,j+1-2ψi,j+ψi-1,jΔy2=ζi,j

# Solution Method

Discretized vorticity transport equation is solved using ADI method which is an implicit and fast method. Also, equation is linearized by lagging u and v velocity components one-time step behind.

x-sweep:

ζi,jn+12-ζi,jnΔt2+(1-ϵx)2ui-1,jnζi+1,jn+12-ui,jnζi,jn+12 Δx+(1+ϵx)2ui,jnζi,jn+12-ui-1,jnζi-1,jn+12 Δx+(1-ϵy)2ui,jn ζi,j+1n-ui,jnζi,jnΔy+(1+ϵy)2ui,jnζi,jn-ui,j-1nζi,j-1nΔy=νζi+1,jn+12-2ζi,jn+12+ζi-1,jn+12Δx2+ζi,j+1n-2ζi,jn+ζi,j-1nΔy2

y-sweep:

ζi,jn+1-ζi,jn+12Δt2+(1-ϵx)2ui-1,jnζi+1,jn+12-ui,jnζi,jn+12 Δx+(1+ϵx)2ui,jnζi,jn+12-ui-1,jnζi-1,jn+12 Δx+(1-ϵy)2ui,jn ζi,j+1n+1-ui,jnζi,jn+1Δy+(1+ϵy)2ui,jnζi,jn+1-ui,j-1nζi,j-1n+1Δy=νζi+1,jn+12-2ζi,jn+12+ζi-1,jn+12Δx2+ζi,j+1n+1-2ζi,jn+1+ζi,j-1n+1Δy2

Each equation has 3 unknowns and can be solved by Thomas algorithm. ADI steps in tridiagonal form

x-sweep:

-Aiζi-1,jn+12+Biζi,jn+12-Ciζi+1,jn+12=Di

Coefficients A, B, C, D are

Ai=12dx+121+ϵxcrxi-1,j

Bi=1+dx+12ϵxcrxi,j

Ci=12dx-121-ϵxcrxi+1,j

Di=12dy-121-ϵycryi,j+1ζi,j+1n+1-dy-12ϵycryi,jζi,jn+12dy+121+ϵycryi,j-1nζi,j-1n

Where

crxi,j=ui,jnΔtΔx  ,  cryi,j=vi,jnΔtΔy  ,  dx=νΔtΔx2  ,  dy=νΔtΔy2

y-sweep:

-Aiζi,j-1n+1+Biζi,jn+1-Ciζi,j+1n+1=Dj

Coefficients A. B. C. D are

Aj=12dy+121+ϵycryi,j-1

Bj=1+dy+12ϵycryi,j

Cj=12dy-121-ϵycryi,j+1

Dj=12dx-121-ϵxcrxi+1,jζi+1,jn+12+1-dx-12ϵxcrxi,jζi,jn+12+12dx+121+ϵxcrxi-1,jζi-1,jn+12

Where

crxi,j=ui,jnΔtΔx  ,  cryi,j=vi,jnΔtΔy  ,  dx=νΔtΔx2  ,  dy=νΔtΔy2

Finally, for solution of discretized Poisson equation Point Successive Over-Relaxation method is used

ψi,jn+1=ψi,jn+ωRi,jn+1

Where

Ri,jn+1=14(ui+1,jn+ui-1,jn+1+ui,j+1n+ui,j-1n+1-4ui,jn+Δ2ζi,jn)

And

1<ω<2

# Domain and Boundary Conditions

Figure 1: Computational Domain

Computational domain has dimensions of 0.01 meter to 0.01 meter and it is divided to 100x100 cells with constant spacing. Upper edge is moving with velocity Up , all the other edges are solid boundaries.

Imposing no slip condition on velocities and considering a sloid surface as a streamline whose values would be constants

Bottom wall: ui,1=0 ;vi,1=0 ; ψi,1=c1=0

Left wall: u1,j=0 ;v1,j=0 ;ψ1,j=c2=0

Right wall: uN,j=0 ;vN,j= 0 ;ψN,j=c3=0

Upper wall: ui,N=Up ;vi,N=0 ;ψi,N=c4=0

There are no boundary conditions for vorticity, but it should be approximated using velocity and stream function values at every iteration.

For left wall:

ψ1,j=c1=0

Substituting above relation to Poisson equation

∂2ψ∂x2+∂2ψ∂y21,j=∂2ψ∂x21,j=-ζ1,j

Using Taylor series expansion for stream function

ψ2,j=ψ1,j+∂ψ∂x​1,jΔx+∂2ψ∂x2​1,jΔx22+…

Where, by definition,

∂ψ∂x​1,j=-v1,j=0

Solving for the second derivative

∂2ψ∂x2​1,j=2(ψ2,j-ψ1,j)Δx2+O(Δx)

Boundary value of vorticity along left wall can now be expressed as

ζ1,j=2(ψ1,j-ψ2,j)Δx2+O(Δx)

Similarly, for right and bottom walls

ζ1,j=2(ψN,j-ψN-1,j)Δx2+O(Δx)

ζi,1=2(ψi,1-ψi,2)Δy2+O(Δx)

Vorticity at upper wall is a little bit different since that wall is moving

Taylor series expression for stream function at upper wall

ψi,N-1=ψi,N+∂ψ∂y​i,NΔy+∂2ψ∂y2​i,NΔy22+…

Where

∂ψ∂y​i,N=Up  ,  ∂2ψ∂y2i,N=-ζi,N

Vorticity is expressed at upper wall such as

ζi,N=2(ψi,N-ψi,N-1)Δy2-2UpΔy

# Initial Conditions

Initial condition for velocity can be defined by interpolation from the moving plate towards interior points or equal to Up everywhere or zeros everywhere. Considering final solution and velocity magnitudes, setting zero velocity at interior points is the best option. The same thing is valid for stream function values, too. So initial values are set as

At interior points

ui,j=0 , vi,j=0

ψi,j=0

Initial values are calculated at interior points for vorticity using velocity values at boundaries and interior points such as

ζ=∂v∂x-∂u∂y

# Stability

Theoretically there is no stability restrictions on time step for an implicit scheme. Still the courant number may be a criterion for maximum time step size. The effect of time step is observed and discussed in the Results section.

For Poisson equation, because it is an elliptic problem, just the final solution is concerned and there is no real time derivative in the discretized equation. In this case, over-relaxation parameter will be adjusted for best solution and fastest convergence.

# Error Definitions

Since there is no an analytical solution or reference value, error must be defined as relative error. Also, the problem is steady, which means change of ζ in time should go to zero. To represent change of ζ in time relatively we need a reference value.  This value may be chosen as old ζ values (from previous iteration). The problem with this approach is that we defined initial ζ values as zero on interior points, which gives division by zero. Other option for reference value is taking the biggest absolute ζ value from previous iteration. This option is better, however, finding biggest value of 100x100 matrix is computationally intense especially at every iteration. It is known that biggest absolute ζ values are located at upper moving wall. So, average of the ζ zeta values at upper wall is taken as a reference value.

Reference value

reference=i=1Nζi,NN

ErrorADI=i=1Nj=1Nζi,j-ζi,joldreferenceN2

Error definition for stream function values is easier since it is an elliptic problem, the residual should go to zero as iteration proceeds. We can define error such as

ErrorPSOR=i=1Nj=1Nψi,jN2

# Solution Algorithm

- Construct Grid

- Initialize Solution Variables

- Start Solution Loop

  - Impose and calculate boundary conditions at every iteration

  - Evaluate ADI coefficients

  - Solve ADI with

    - Perform x-sweep for every j index and get half time step solution

    - Perform y-sweep for every I index using half time step solution and get full time step solution

  - Solve PSOR with 20 iteration for every solution loop using ω=1.8

  - Evaluate new velocity components using ψ values from PSOR solution

  - Evaluate error at each solution loop

- End Solution Loop

- Output the variables x, y, u, v, ζ,  ψ

- Calculate the drag and print it out

# Results and Discussion

Figure 2: Effect of Over-Relaxation parameter on PSOR convergence

Figure 2 represent the residual(R) behavior of PSOR solution from previous homework. To get best convergence ω would be chosen 1.95. However, that value is for the first iteration and boundary values are changing at every iteration. Thus, it is logical to not use the extreme ω value. It seems ω 1.8 gives enough convergence speed and stability throughout the solution.

In addition, there is no need to get a converged solution for PSOR at each program loop. The intermediate steps do not represent a valid solution so there is no point to put computational effort on to solution. 20 iterations on PSOR for each program loop seem enough to get a solution.

Figure 3: Effect of Time Step on Convergence Re=102

Another parameter to adjust is time step to use in ADI scheme. Although, there is no theoretical limitation on time step, the solution diverges at Re=102 and time step greater than 7×10-3. In addition, taking larger time steps results in more oscillatory behavior. This can also be seen from figure 3. For fast convergence and good accuracy time step is calculated for different Up as

Δt=0.5×ΔxUp

Figure 4: Streamlines Re=104

Figure 5: Velocity Vectors Re=104

Figure 6: Convergence History Re=104

First case is with Up=1 m/s and resultant Reynold number is Re=104. There are 3 separation regions that can be seen in figure 3 and as velocity vectors in figure 4. It takes around 24000 iterations to get a relative error below 10-7.

Figure 7: Streamlines Re=103

Figure 8: Velocity Vectors Re=103

Figure 9: Convergence History Re=103

When Re=103  separation only occurs at bottom corners as indicated in figures 7 and 8. The convergence is much faster in this case, 10 thousand iteration is enough to get a relative error below 10-7.

Figure 10: Streamlines Re=5×102

Figure 11: Velocity Vectors Re=5×102

Figure 12: Convergence History Re=5×102

Figure 10 and 11 shows the stream function behavior and velocity vectors, as can be seen the separation at bottom left corner is very small (Re=5×102) compared to previous case. Also, the convergence is getting faster, figure 12 shows that around 7 thousand of iteration gives relative error below 10-7.

Figure 13: Streamlines Re=2×102

Figure 14: Velocity Vectors Re=2×102

Figure 15: Convergence History Re=2×102

With reduced further reduction of Reynolds number to Re=2×102 eliminates the separation at bottom left corner. Again, the convergence is faster than previous case.

Figure 16: Streamlines Re=102

Figure 17: Velocity Vectors Re=102

Figure 18: Convergence History Re=102

In the final case where Re=102 , there is almost no separation of flow and the convergence is fastest with around 2500 iterations.

| Up [m/s] | Re | Max ψ value at center | Max ψ at Separation | Drag Force [N/m] |
| --- | --- | --- | --- | --- |
| 1 | 1.0E+04 | -5.350E-04 | 1.934E-05 | 7.553E-02 |
| 0.1 | 1.0E+03 | -9.029E-05 | 3.892E-07 | 3.347E-03 |
| 0.05 | 5.0E+02 | -4.828E-05 | 1.069E-07 | 1.346E-03 |
| 0.02 | 2.0E+02 | -2.006E-05 | 8.692E-09 | 4.302E-04 |
| 0.01 | 1.0E+02 | -1.006E-05 | 9.109E-10 | 1.924E-04 |
| Table 1: Results |  |  |  |  |

Overall results are given in table 1.  Higher velocity at upper plates gives bigger stream function value at the core. The reason for that is stream function value at the upper wall is set to zero and stream function values calculated with integration of velocity differences. Bigger the velocity difference bigger the stream function values (The negative sigh at center ψ values are a result of notation). In addition, the core ψ value is affected by presence of separation. Stronger separation results in bigger stream function value.

As the definition of Reynolds Number, ratio of inertial forces to viscous forces, indicates, in higher Re flows separation becomes stronger.

At separation regions velocity vector directions are changes more than no separation cases, this might be the reason of convergence speed difference. At Re=104 convergence is achieved with 24000 iterations and at Re=102 solution converged at 3000 iterations. Although we used upwinding method, changing solution direction through solution makes convergence slower but it prevents divergence.

Drag Force is calculated as unit width using trapezoidal integration at upper moving plate. Using shear stress relation

τ=μdudy

Where μ is taken as 10-3  and  ρ=1000 kg/m3

It is expected to have higher drag with higher plate velocity. As the Reynolds number gets bigger the relation between Up and Drag Force becomes highly nonlinear.

# Program Listing

program CavityFlow c..Taha Yaşar Demir / 1881978 c..CE-580 HomeWork #7 parameter (mx=101) common/grd/ x(mx,mx),y(mx,mx),dydx,N,rL,epsx(mx,mx),epsy(mx,mx) common/flw/ u(mx,mx),v(mx,mx),psi(mx,mx),zeta(mx,mx) common/par/ u_p, visc, dt, time, crx(mx,mx), cry(mx,mx),dx,dy common/err/ Erv, Ers, R(mx,mx) common/tho/ a(mx),b(mx),c(mx),d(mx) real tolerance integer iter open(22,file="erroradi.dat") call grid call init Ers = 1. Erv = 1. iter= 0. tolerance = 1e-7 c do while(Erv.gt.tolerance) do m=1,20000 iter = iter + 1 call boundary call evalcoef call ADI call psor call velocity call error(iter) print*, iter,Erv,Ers enddo call output call dragcal close(22) close(33) stop end c----------------------------------------------------------------------- subroutine grid parameter (mx=101) common/grd/ x(mx,mx),y(mx,mx),dydx,N,rL,epsx(mx,mx),epsy(mx,mx) common/flw/ u(mx,mx),v(mx,mx),psi(mx,mx),zeta(mx,mx) common/par/ u_p, visc, dt, time, crx(mx,mx), cry(mx,mx),dx,dy common/err/ Erv, Ers, R(mx,mx) common/tho/ a(mx),b(mx),c(mx),d(mx) rL = 0.01 !m N = 101 dydx = rL/(N-1) do j=1,N x(1,j) = 0. do i=2,N-1 x(i,j) = x(i-1,j) + dydx enddo x(N,j) = rL enddo do i=1,N y(i,1) = 0. do j=2,N-1 y(i,j) = y(i,j-1) + dydx enddo y(i,N) = rL enddo return end c----------------------------------------------------------------------- subroutine init parameter (mx=101) common/grd/ x(mx,mx),y(mx,mx),dydx,N,rL,epsx(mx,mx),epsy(mx,mx) common/flw/ u(mx,mx),v(mx,mx),psi(mx,mx),zeta(mx,mx) common/par/ u_p, visc, dt, time, crx(mx,mx), cry(mx,mx),dx,dy common/err/ Erv, Ers, R(mx,mx) common/tho/ a(mx),b(mx),c(mx),d(mx) time = 0. u_p = 0.01 ! 0.01-0.02-0.05-0.1-1 visc = 1E-6 dt = 0.1*(dydx/u_p) call boundary do i=2,N-1 do j=2,N-1 u(i,j) = 0. v(i,j) = 0. psi(i,j) = 0. zeta(i,j)= (v(i,j)-v(i-1,j))/dydx - (u(i,j)-u(i-1,j))/dydx enddo enddo return end c----------------------------------------------------------------------- subroutine boundary parameter (mx=101) common/grd/ x(mx,mx),y(mx,mx),dydx,N,rL,epsx(mx,mx),epsy(mx,mx) common/flw/ u(mx,mx),v(mx,mx),psi(mx,mx),zeta(mx,mx) common/par/ u_p, visc, dt, time, crx(mx,mx), cry(mx,mx),dx,dy common/err/ Erv, Ers, R(mx,mx) common/tho/ a(mx),b(mx),c(mx),d(mx) do i=2,N-1 psi(i,1) = 0. psi(i,N) = 0. u(i,1) = 0. u(i,N) = u_p v(i,1) = 0. v(i,N) = 0. zeta(i,1)= 2*(psi(i,1)-psi(i,2))/(dydx**2) zeta(i,N)= (2*(psi(i,N)-psi(i,N-1))/(dydx**2)) - 2*u_p/dydx enddo do j=1,N psi(1,j) = 0. psi(N,j) = 0. u(1,j) = 0. u(N,j) = 0. v(1,j) = 0. v(N,j) = 0. zeta(1,j)= 2*(psi(1,j)-psi(2,j))/(dydx**2) zeta(N,j)= 2*(psi(N,j)-psi(N-1,j))/(dydx**2) enddo return end c----------------------------------------------------------------------- subroutine evalcoef parameter (mx=101) common/grd/ x(mx,mx),y(mx,mx),dydx,N,rL,epsx(mx,mx),epsy(mx,mx) common/flw/ u(mx,mx),v(mx,mx),psi(mx,mx),zeta(mx,mx) common/par/ u_p, visc, dt, time, crx(mx,mx), cry(mx,mx),dx,dy common/err/ Erv, Ers, R(mx,mx) common/tho/ a(mx),b(mx),c(mx),d(mx) do i=1,N do j=1,N if (u(i,j).gt.0.) then epsx(i,j) = 1. else epsx(i,j) =-1. endif if (v(i,j).gt.0.) then epsy(i,j) = 1. else epsy(i,j) =-1. endif crx(i,j) = u(i,j)*dt/dydx cry(i,j) = v(i,j)*dt/dydx dx = visc*dt/(dydx**2) dy = visc*dt/(dydx**2) enddo enddo return end c----------------------------------------------------------------------- subroutine ADI parameter (mx=101) common/grd/ x(mx,mx),y(mx,mx),dydx,N,rL,epsx(mx,mx),epsy(mx,mx) common/flw/ u(mx,mx),v(mx,mx),psi(mx,mx),zeta(mx,mx) common/par/ u_p, visc, dt, time, crx(mx,mx), cry(mx,mx),dx,dy common/err/ Erv, Ers, R(mx,mx) common/tho/ a(mx),b(mx),c(mx),d(mx) c...x_sweep do j=2,N-1 call x_sweep(j) enddo c...y_sweep do i=2,N-1 call y_sweep(i) enddo return end c----------------------------------------------------------------------- subroutine x_sweep(j) parameter (mx=101) common/grd/ x(mx,mx),y(mx,mx),dydx,N,rL,epsx(mx,mx),epsy(mx,mx) common/flw/ u(mx,mx),v(mx,mx),psi(mx,mx),zeta(mx,mx) common/par/ u_p, visc, dt, time, crx(mx,mx), cry(mx,mx),dx,dy common/err/ Erv, Ers, R(mx,mx) common/tho/ a(mx),b(mx),c(mx),d(mx) do i=2,N-1 if(i.eq.2) then a(i) = 0. b(i) = 1 + dx + 0.5*epsx(i,j)*crx(i,j) c(i) = -0.5*(dx - 0.5*(1-epsx(i,j))*crx(i+1,j)) d(i) = 0.5*(dx + 0.5*(1+epsx(i,j))*crx(i-1,j))*zeta(i-1,j) & + 0.5*(dy - 0.5*(1-epsy(i,j))*cry(i,j+1))*zeta(i,j+1) & + (1-dy-0.5*epsy(i,j)*cry(i,j))*zeta(i,j) & + 0.5*(dy + 0.5*(1+epsy(i,j))*cry(i,j-1))*zeta(i,j-1) elseif(i.eq.N-1) then a(i) = -0.5*(dx + 0.5*(1+epsx(i,j))*crx(i-1,j)) b(i) = 1 + dx + 0.5*epsx(i,j)*crx(i,j) c(i) = 0. d(i) = 0.5*(dx - 0.5*(1-epsx(i,j))*crx(i+1,j))*zeta(i+1,j) & + 0.5*(dy - 0.5*(1-epsy(i,j))*cry(i,j+1))*zeta(i,j+1) & + (1-dy-0.5*epsy(i,j)*cry(i,j))*zeta(i,j) & + 0.5*(dy + 0.5*(1+epsy(i,j))*cry(i,j-1))*zeta(i,j-1) else a(i) = -0.5*(dx + 0.5*(1+epsx(i,j))*crx(i-1,j)) b(i) = 1 + dx + 0.5*epsx(i,j)*crx(i,j) c(i) = -0.5*(dx - 0.5*(1-epsx(i,j))*crx(i+1,j)) d(i) = 0.5*(dy - 0.5*(1-epsy(i,j))*cry(i,j+1))*zeta(i,j+1) & + (1-dy-0.5*epsy(i,j)*cry(i,j))*zeta(i,j) & + 0.5*(dy + 0.5*(1+epsy(i,j))*cry(i,j-1))*zeta(i,j-1) endif enddo call THOMAS(2,N-1,a,b,c,d) do i=2,N-1 ! extract the solution from thomas algorithm zeta(i,j) = d(i) enddo return end c----------------------------------------------------------------------- subroutine y_sweep(i) parameter (mx=101) common/grd/ x(mx,mx),y(mx,mx),dydx,N,rL,epsx(mx,mx),epsy(mx,mx) common/flw/ u(mx,mx),v(mx,mx),psi(mx,mx),zeta(mx,mx) common/par/ u_p, visc, dt, time, crx(mx,mx), cry(mx,mx),dx,dy common/err/ Erv, Ers, R(mx,mx) common/tho/ a(mx),b(mx),c(mx),d(mx) do j=2,N-1 if(j.eq.2) then a(j) = 0. b(j) = 1 + dy + 0.5*(epsy(i,j))*cry(i,j) c(j) = -0.5*(dy - 0.5*(1-epsy(i,j))*cry(i,j+1)) d(j) = 0.5*(dy + 0.5*(1+epsy(i,j))*cry(i,j-1))*zeta(i,j-1) & +0.5*(dx - 0.5*(1-epsx(i,j))*crx(i+1,j))*zeta(i+1,j) & + (1-dx-0.5*epsx(i,j)*crx(i,j))*zeta(i,j) & +0.5*(dx + 0.5*(1+epsx(i,j))*crx(i-1,j))*zeta(i-1,j) elseif(j.eq.N-1) then a(j) = -0.5*(dy + 0.5*(1+epsy(i,j))*cry(i,j-1)) b(j) = 1 + dy + 0.5*(epsy(i,j))*cry(i,j) c(j) = 0. d(j) = 0.5*(dy - 0.5*(1-epsy(i,j))*cry(i,j+1))*zeta(i,j+1) & +0.5*(dx - 0.5*(1-epsx(i,j))*crx(i+1,j))*zeta(i+1,j) & + (1-dx-0.5*epsx(i,j)*crx(i,j))*zeta(i,j) & +0.5*(dx + 0.5*(1+epsx(i,j))*crx(i-1,j))*zeta(i-1,j) else a(j) = -0.5*(dy + 0.5*(1+epsy(i,j))*cry(i,j-1)) b(j) = 1 + dy + 0.5*(epsy(i,j))*cry(i,j) c(j) = -0.5*(dy - 0.5*(1-epsy(i,j))*cry(i,j+1)) d(j) = 0.5*(dx - 0.5*(1-epsx(i,j))*crx(i+1,j))*zeta(i+1,j) & + (1-dx-0.5*epsx(i,j)*crx(i,j))*zeta(i,j) & +0.5*(dx + 0.5*(1+epsx(i,j))*crx(i-1,j))*zeta(i-1,j) endif enddo call THOMAS(2,N-1,a,b,c,d) do j=2,N-1 zeta(i,j) = d(j) enddo return end c----------------------------------------------------------------------- subroutine psor parameter (mx=101) common/grd/ x(mx,mx),y(mx,mx),dydx,N,rL,epsx(mx,mx),epsy(mx,mx) common/flw/ u(mx,mx),v(mx,mx),psi(mx,mx),zeta(mx,mx) common/par/ u_p, visc, dt, time, crx(mx,mx), cry(mx,mx),dx,dy common/err/ Erv, Ers, R(mx,mx) common/tho/ a(mx),b(mx),c(mx),d(mx) real omega,sum open(33,file="errorpsor.dat") omega = 1.8 ! over-relaxation parameter sum = 0. do k=1,20 do j=2,N-1 do i=2,N-1 R(i,j) = 0.25*(psi(i+1,j)+psi(i-1,j)+psi(i,j+1)+psi(i,j-1) & - 4*psi(i,j) + (dydx**2)*zeta(i,j)) psi(i,j) = psi(i,j) + omega*R(i,j) sum = sum + abs(R(i,j)) enddo enddo sum = sum/((N-2)**2) write(33,*) k,sum enddo return end c----------------------------------------------------------------------- subroutine velocity parameter (mx=101) common/grd/ x(mx,mx),y(mx,mx),dydx,N,rL,epsx(mx,mx),epsy(mx,mx) common/flw/ u(mx,mx),v(mx,mx),psi(mx,mx),zeta(mx,mx) common/par/ u_p, visc, dt, time, crx(mx,mx), cry(mx,mx),dx,dy common/err/ Erv, Ers, R(mx,mx) common/tho/ a(mx),b(mx),c(mx),d(mx) do i=2,N-1 do j=2,N-1 u(i,j) = (psi(i,j+1) - psi(i,j))/dydx v(i,j) =-(psi(i+1,j) - psi(i,j))/dydx enddo enddo return end c----------------------------------------------------------------------- subroutine error(iteration) parameter (mx=101) common/grd/ x(mx,mx),y(mx,mx),dydx,N,rL,epsx(mx,mx),epsy(mx,mx) common/flw/ u(mx,mx),v(mx,mx),psi(mx,mx),zeta(mx,mx) common/par/ u_p, visc, dt, time, crx(mx,mx), cry(mx,mx),dx,dy common/err/ Erv, Ers, R(mx,mx) common/tho/ a(mx),b(mx),c(mx),d(mx) real adier , psorer, zetaold(mx,mx), psiold(mx,mx), sum integer iteration adier = 0. psorer= 0. sum = 0. do i=1,N sum = sum + abs(zeta(i,N)) enddo sum = sum/N c.. L2 normalization is used for vorticity and stream function values if(iteration.eq.1) then ! store the previous zeta and psi values do j=1,N do i=1,N zetaold(i,j) = zeta(i,j) psiold(i,j) = psi(i,j) enddo enddo print*, else do j=1,N do i=1,N adier=adier+abs((zeta(i,j)-zetaold(i,j))/sum) zetaold(i,j) = zeta(i,j) ! update the old values for next iteration psorer = psorer+abs(psi(i,j)-psiold(i,j)) psiold(i,j) = psi(i,j) enddo enddo Erv = adier/(N**2) ! Vorticity transport equation error Ers = psorer/(N**2)! Stream function solution errror write(22,*) iteration,Erv,Ers endif return end c----------------------------------------------------------------------- subroutine output parameter (mx=101) common/grd/ x(mx,mx),y(mx,mx),dydx,N,rL,epsx(mx,mx),epsy(mx,mx) common/flw/ u(mx,mx),v(mx,mx),psi(mx,mx),zeta(mx,mx) common/par/ u_p, visc, dt, time, crx(mx,mx), cry(mx,mx),dx,dy common/err/ Erv, Ers, R(mx,mx) common/tho/ a(mx),b(mx),c(mx),d(mx) open(11,file='var.tec',form='formatted') write(11,*) ' variables="x","y","zeta","psi","u","v" ' write(11,*) ' zone i=',N, 'j=',N do j=1,N do i=1,N write(11,'(8E12.4)') x(i,j),y(i,j),zeta(i,j),psi(i,j), + u(i,j),v(i,j) enddo enddo return end c----------------------------------------------------------------------- subroutine dragcal parameter (mx=101) common/grd/ x(mx,mx),y(mx,mx),dydx,N,rL,epsx(mx,mx),epsy(mx,mx) common/flw/ u(mx,mx),v(mx,mx),psi(mx,mx),zeta(mx,mx) common/par/ u_p, visc, dt, time, crx(mx,mx), cry(mx,mx),dx,dy common/err/ Erv, Ers, R(mx,mx) common/tho/ a(mx),b(mx),c(mx),d(mx) real mu,rho,shear,drag rho = 1000. mu = visc * rho drag = 0. do i=2,N shear = mu*(u(i,N)-u(i,N-1))/dydx drag = drag + shear*(x(i,N)-x(i-1,N)) enddo print*, "2-D drag force = ", drag return end c----------------------------------------------------------------------- subroutine THOMAS(il,iu,aa,bb,cc,ff) c............................................................ c Solution of a tridiagonal system of n equations of the form c A(i)*x(i-1) + B(i)*x(i) + C(i)*x(i+1) = R(i) for i=il,iu c the solution X(i) is stored in F(i) c A(il-1) and C(iu+1) are not used c A,Bb,C,R are arrays to bbe provided bby the user c............................................................ parameter (mx=101) dimension aa(mx),bb(mx),cc(mx),ff(mx),tmp(mx) tmp(il)=cc(il)/bb(il) ff(il)=ff(il)/bb(il) ilp1 = il+1 do i=ilp1,iu z=1./(bb(i)-aa(i)*tmp(i-1)) tmp(i)=cc(i)*Z ff(i)=(ff(i)-aa(i)*ff(i-1))*z enddo iupil=iu+il do ii=ilp1,iu i=iupil-ii ff(i)=ff(i)-tmp(i)*ff(i+1) enddo return end

Fortran Code